电 子 科 技 大 学

学术学位研究生文献综述报告

攻读学位级别： □博士        硕士

培养方式： 全日制      □非全日制

专业学位类别及领域：      控制科学与工程

学        院：            自动化工程学院

学        号：             202421060724

姓        名：                 华羽霄

论文题目：     基于多模态逐级物理约束的无

人机自主电磁超声探测技术研究

校内指导教师：              高斌  教授

指导教师阅读后签字：

填   表日期：    2026    年   3   月    19   日

电子科技大学研究生院

# Abstract

With the continuous expansion of modern industrial equipment and large-scale infrastructure, non-destructive testing (NDT) technology based on multi-rotor drones has gradually become an important means of structural health monitoring due to its high mobility and flexibility. Traditional drone visual inspection, due to sensor limitations, can only identify visible surface defects and struggles to acquire damage information from within the structure and to the subsurface. Electromagnetic ultrasonic testing (EMAT), as a non-contact/weak-contact technology that does not require acoustic coupling agents, is highly sensitive to subsurface defects. Currently, researchers have made significant progress in optimizing the magnetic field and enhancing signal resonance for its application in thickness measurement. Integrating electromagnetic ultrasonic probes into drone platforms represents a cutting-edge development direction that breaks through the depth limitations of traditional visual inspection.

However, when performing contact ultrasonic testing, drones face core challenges such as large airflow impact at the moment of contact and the difficulty in maintaining stable dynamic contact force. Based on the latest research at home and abroad, UAV contact detection systems are undergoing profound evolution: from early passive compliant buffer mechanisms such as end-effector spring damping, they are gradually developing into complex dynamic coupling systems relying on fully driven/overdriven platforms and active impedance control, aiming to dynamically adjust and maintain high-precision normal contact force under complex curved surfaces and strong disturbance environments. Despite continuous innovation in control strategies, under actual flight disturbances, relying solely on a single ultrasonic signal or visual information is still insufficient to accurately and stably determine the true contact state between the probe and the object surface, easily leading to misjudgments. Furthermore, existing purely data-driven detection models often lack physical constraints and have insufficient generalization ability under complex conditions.

To address the limitations of existing research in single-modal perception and lack of physical consistency, this project proposes an autonomous electromagnetic ultrasonic detection technology for UAVs based on multimodal hierarchical physical constraints. This framework innovatively constructs a temporal Transformer perception network that integrates electromagnetic ultrasonic echo sequences, UAV pose dynamic residuals, and RGBD local visual ROI features, achieving cross-modal probabilistic characterization of the contact state. Meanwhile, this study embeds UAV contact dynamics models and state transition constraints (such as Hidden Markov Models) as physical priors into the attention mechanism and inference process of deep networks, effectively suppressing high-frequency state jitter. This research aims to address the shortcomings of dynamic contact perception, propelling multi-rotor UAVs towards an integrated intelligent non-destructive testing platform encompassing perception, decision-making, and control, and providing systematic theoretical and engineering support for highly reliable operations in complex high-altitude environments.

# 文献综述

- 无人机无损缺陷检测技术

利用空中平台对大型复杂结构进行无损检测 (NDI) 已成为各行各业日益关注的领域。诸如建筑物、桥梁、石油天然气炼油厂等基础设施需要定期进行全面检测[1]。检测报告用于规划和执行必要的维护工作，以确保结构健康和工人安全。然而，由于设施规模庞大、难以进入、检测人员面临健康风险或其他诸多原因，执行这些检测可能极具挑战性，因此促使企业加大对无人机的投资，将其作为克服这些挑战的替代方案[2]。

2010年，法国PARROT公司推出了其首款四旋翼无人机AR.Drone 1.0，该机配备高清摄像头用于数据采集，并支持Wi-Fi热点功能，通过移动设备实现远程控制，并能实时传输视频流[3]。2013年，中国DJI公司研发并发布了首款小型四旋翼无人机Phantom 1.0，内置GPS传感器，具备位置保持功能，便于进行检查作业。该设备还配有运动相机及可调角度支架，使相机与机身紧密集成，且在多风环境中表现出优异的稳定性[4]。2014年，美国3D ROBOTICS公司推出了消费级无人机IRIS和IRIS+，该机型的特色是跟随技术，能够使无人机跟随移动目标并自动对焦[5]。与此同时，像中国的YUNEEC公司、美国的SKYDIO公司、AERO VIRONMENT公司以及瑞士的SENSEFLY公司等也纷纷推出了各自的无人机产品。经过十多年的技术进步，无人机的性能已达到新的高度。当前具有代表性的型号包括中国DJI公司的M300 RTK、法国PARROT公司的Anafi，以及美国AEROVIRONMENT公司的QuantixRecon等。

无人机的自主性可以帮助企业缩短检测时间和降低成本。此外，使用无人机还可以减少检测所需的人员数量，并提高人员安全。最后，无人机可以为难以到达或危险区域的检测提供安全可靠的解决方案[6]。尽管基于无人机的无损检测技术近年来取得了长足发展，能够可靠地检测缺陷，但仍存在一些局限性和挑战需要解决。

- 电磁超声厚度测量的研究现状

电磁超声工作条件是在非耦合条件下，其超声波在工件表面产生，相较于传统的压电式超声换能器能够避免工件表面粗糙、耦合层声速不均一所造成的声速测量误差，并且声速测量对换能器灵敏度要求低，故 EMAT 的相关研究往往围绕厚度测量方向[7]。

涂君等人[8]提出了磁力可控电磁超声换能器来测量试件厚度。通过永磁体与电磁铁组合的方式控制磁场强度，其中通过控制电磁铁线圈通电方向来控制电磁铁磁场方向，进而实现磁场总体的增强和削弱。当线圈未通电时，磁场仅由永磁体部分提供。Chen W 等人[9]提出了一种超声纵波共振方法，该方法基于材料的表面约束机制利用激光电磁声换能器（Laser-EMAT）来测量金属壁的厚度，能够准确检测出厚度从 3.0 毫米减薄至 0.5mm 的阶梯状铝板。并且能够实现在 500°C 的温度下，当 EMAT 提离距离为 5.0mm 时，对铝薄板的厚度进行快速检测，误差为 3.40%。Kang, L 等人[10]提出一种基于双电磁超声换能器的超声波最佳单向激发的板材厚度测量方法。通过使用双线圈电磁超声换能器，通过适当相位信号驱动线圈的干涉机制实现 SH 超声导波单向激发，再通过计算频散曲线得到对应板材厚度，测量误差低于 0.8%。郭伟等人[11]提出了一种基于横波谐振频谱测厚的激光电磁超声技术。以铝合金 5083、不锈钢 304、镍基合金 GH4169 这 3 种金属材料为检测对象，开展了常温、高温金属试样谐振测厚实验。研究结果表明，当上述 3 种金属材料试样厚度为 0.5mm~3.0mm 时，谐振法测厚的最大偏差不超过 2%、 6%、 4%。苟浩瑞等人[12]提出了多磁铁对称分布型 EMAT，能实现更小的磁铁体积，产生更强的表面剩磁强度。采用在硅钢表面开槽的方式限制涡流形成的区域，解决了涡流对测量的影响。多磁铁对称分布型 EMAT 具有较高的信噪比且信号质量要优于单磁铁型 EMAT。Pei C 等人[13]将单一永磁体结构改为双永磁体垂直方向反向排布实现了磁场优化，并应用于曲折线圈的导波激励与接收，接收信号幅值提升一倍。

- 无人机非侵入接触式检测技术

近年来，多旋翼无人机在工业无损检测（NDT）领域的应用正经历从传统的视觉巡检向接触式检测的深刻变革。与非接触式视觉巡检不同，接触式NDT（如超声波检测）要求无人机在与被测表面发生物理交互时，能够克服接触瞬间的冲击扰动，并维持稳定、精确的法向接触力。这一过程涉及复杂的运动学与动力学耦合，是当前空中机器人领域的研究热点与难点。纵观近年来的文献，该领域的研究主要沿着“被动柔顺机构设计”、“主动力控与过驱动平台引入”以及“受限空间系统级框架集成”三条主线展开。

在早期的研究中，学者们主要侧重于通过机械结构与末端执行器的设计来缓解接触碰撞带来的扰动。2020年，L.M. González-deSantos等人[14]的研究进一步设计了一种具备碰撞缓解功能的柔性有效载荷。该装置通过弹簧阻尼系统等被动柔顺机构，有效吸收了四旋翼无人机从自由飞行向接触状态过渡时的动能冲击，提升了系统在接触瞬间的鲁棒性。

随着对检测精度和接触力维持要求的不断提高，研究重心逐渐转向先进的主动控制策略。对于超声波无损检测而言，维持稳定且垂直的接触力是获取高质量信号的前提。在精确力控算法方面，2019年，Kocer等人[15]针对天花板等上方结构检测，提出了一种基于非线性模型预测控制（NMPC）与非线性移动视界估计（NMHE）的框架。该方法将接触产生的未知扰动作为外部力进行在线估计并反馈给预测控制器，成功使四旋翼无人机在5.5 N的恒定接触力下稳定获取超声数据。

在硬件平台的进一步演进中，全驱动和过驱动（Over-actuated）无人机凭借独立的推力矢量控制，成为解决复杂面接触难题的另一核心路径。2018年，Anibal Ollero等人[16,17]针对油气工业中严苛的检测环境，提出了一种新型空中机械臂系统AeroX。该方案通过在八旋翼无人机底部搭载具有特定自由度的机械臂，实现了检测探头与管壁的贴合，初步验证了多旋翼平台执行接触式NDT的工程可行性。然而，刚性接触容易导致飞行失稳。2020年，Ryll等人[18]针对未知通用形状表面的接触检测，提出了一种基于视觉的阻抗控制框架，并将其应用于全驱动（Fully-Actuated）六旋翼无人机上。该研究利用全驱动平台位置与姿态解耦的运动学优势，结合阻抗控制算法，使得无人机能够依靠主动力控在复杂的接触动力学环境下动态调节接触力，避免了传统欠驱动四旋翼无人机在施加水平力时必须倾斜机身的问题。2021年，Robert J. Watson等人[19–21] 利用具有全矢量推力能力的Voliro多旋翼无人机，深入探讨了干耦合超声无损检测的空中应用。干耦合超声轮式探头虽无需液体耦合剂，但要求施加约20 N的持续法向压紧力。Voliro不仅能在倾斜和悬垂表面实现精度达0.1 mm的单点厚度测量 ，还通过该5 MHz双晶轮式探头实现了连续滚动的B扫描（B-scan）轮廓获取，显著拓宽了无人机接触式检测的操作维度。针对接触检测中的复杂交互动力学，2024年，Salvatore Marcellini等人[22]提出了一种基于倾转旋翼的半自主无损检测框架。该研究通过将倾转旋翼的全驱动姿态解耦优势与机械臂的柔顺力控相融合，系统在接触阶段通过这种双重阻抗控制架构，动态吸收接触瞬间的物理冲击扰动，从而在复杂的外界环境下实现超声探头恒定法向接触力的精准维持与调整。2025年，He[23]提出一种统一的空中操作框架，推动无人机从被动感知向主动物理交互演进 。其核心包含三部分：1) 混合运动-力控制模块，实现与环境动态且柔顺的接触 ；2) 接触感知轨迹规划器，在约束下生成安全轨迹，支持空中书写等连续任务 ；3) 以末端执行器为中心的控制接口，将底层飞行控制与高级策略学习成功解耦。实验证明，该框架显著提升了无人机的操作精度、鲁棒性与跨平台泛化能力 。

步入系统级应用阶段，多传感器融合与针对极端受限空间的工程化部署成为了最新趋势。针对复杂危险的密闭空间（如管道内部、储罐），2025年，Flyability公司推出了Elios 3 UT笼式无人机系统[24–26]。该系统将Cygnus双晶超声探头、智能机械臂与LiDAR传感器高度集成在保护笼内，能够在无需搭建脚手架的情况下安全深入受限空间，在贴壁飞行时实时获取精确的A扫描数据，极大地提升了检测效率与安全性。

真实的高空检测环境中，持续的风场扰动和表面曲率突变会导致接触判定产生高频抖动，进而引发控制闭环失效。面对深度学习在连续在线学习中普遍存在的“灾难性遗忘”难题，2025年，Liu等人[27]提出了结合持续反向传播（CBP）与PINN的在线状态估计框架。CBP算法通过引入“效用度量”（Utility Metric），持续监控网络神经元的历史贡献度，动态锁定成熟单元并仅选择性重置效用极低的非活性单元，同时严格限制权重的无序增长。该机制使得无人机能够在保留离线学习到的刚体动力学规律的前提下，通过在线小批量实时流数据（Streaming Data）不断吸收并自适应外部未知气流的动态演变特征。

综上所述，多旋翼无人机接触式无损检测技术已从初期的末端被动缓冲设计，发展为高度依赖过驱动平台与复杂动力学控制（如阻抗控制）的深度耦合系统。未来的研究将更加聚焦于高并发环境下的系统鲁棒性、多传感器协同目标位姿估计，以及复杂曲面下的高精度阻抗调节，以期实现超声探头等NDT设备在更广泛工业场景下的高可靠性作业。

- 物理约束Transformer

纯数据驱动模型（如Vanilla DNN）具有“黑盒”特性，极易学习到数据中的伪关系（Spurious Relationships），在面临分布外（OOD）的复杂扰动时容易输出违背物理常理的预测[28]。近年来，物理信息神经网络（PINN）与Transformer架构的深度融合，通过将物理演化规律作为归纳偏置（Inductive Bias）嵌入大模型，为无人机多模态感知与自主决策开辟了新范式。

为克服此局限，物理信息神经网络（PINN）利用自动微分技术，将常微分或偏微分方程作为物理残差项直接嵌入损失函数中，发挥了强大的正则化作用。面对参数化与非参数化不确定性，2023年，Sanyal等人[29]提出的RAMP-Net框架创新性地采用了混合训练策略：物理损失负责捕获理想动力学方程以维持系统鲁棒性，而数据损失则自适应处理未建模的残余扰动。在更为复杂的先进空气动力学（如地面效应、旋翼间干扰）建模中，通过局部单调性损失（Local Monotonicity Loss）将“动量守恒定律”作为软约束嵌入，并辅以循环退火调度器（Cyclical Annealing Scheduler）动态调整物理损失权重，模型展现了卓越的泛化能力。2024年，Gu等人[30]通过强制网络输出严格逼近包含质量、科里奥利力等项的6-DOF刚体运动学和牛顿-欧拉动力学方程，PINN能有效过滤高频传感器噪声，并高保真地填补稀疏的未观测状态。

基于多层感知机（MLP）的PINN缺乏对时序上下文的显式建模，在处理包含高频特征或多尺度耦合的物理场（如电磁超声的弹性动力学传播）时，极易陷入局部极小值并产生过度平滑的失效模式（Failure Modes）。为解决此瓶颈，2023年，Zhao等人[31]提出PINNsFormer框架，实现了物理约束网络从静态点评估向动态序列推演的跨越。该架构利用“伪序列生成器（Pseudo Sequence Generator）”将逐点时空输入外推为等距伪序列，随后通过时空混合器（Spatio-Temporal Mixer）映射至高维空间，最后利用Transformer的多头注意力机制精准捕捉全局时间依赖。更具革命性的是，PINNsFormer引入了基于实数傅里叶变换理论的Wavelet激活函数ω1sinx+ω2cosx，理论证明它是无限宽网络的通用近似器，能够前瞻性地分解并拟合复杂信号的高频振荡特征。

单一的电磁超声信号在无人机飞行扰动与姿态变化下极易失真，产生接触状态的误判。因此，必须深度融合红绿蓝（RGB）视觉和激光雷达（LiDAR）等多源感知信息。针对异构传感器采样频率不一导致的时间错位，2026年，Xie等人[32]提出MPFusionNet，结合动态时间规整（DTW）和三次样条插值，将低频视觉与高频雷达信号重采样至统一时序尺度上，确保了特征融合前的时间强连贯性。在空间维度，该网络将GPS坐标转化为几何感知先验（Geometry-aware Priors），并利用基于PerceiverIO的Transformer交叉注意力机制（Cross-attention），与多模态序列进行深层交互，在高度遮挡的非视距（NLOS）环境下依然维持了极高的预测鲁棒性。

参考文献：

[1]	QU Z, JIANG P, ZHANG W. Development and Application of Infrared Thermography Non-Destructive Testing Techniques[J/OL]. Sensors, 2020. DOI:10.3390/s20143851.

[2]	BALL R J, ALMOND D P. The detection and measurement of impact damage in thick carbon fibre reinforced laminates by transient thermography[J/OL]. NDT & E International, 1998, 31(3): 165-173. DOI:10.1016/S0963-8695(97)00052-2.

[3]	ABUGHALI A, HABASH O, ELSHAMY A, 等. Design and Analysis of a Linear Controller for Parrot AR Drone 2.0[C/OL]//2022 International Conference on Electrical and Computing Technologies and Applications (ICECTA). 2022: 113-117. DOI:10.1109/ICECTA57148.2022.9990127.

[4]	TADDIA Y, GONZÁLEZ-GARCÍA L, ZAMBELLO E, 等. Quality Assessment of Photogrammetric Models for Façade and Building Reconstruction Using DJI Phantom 4 RTK[J].

[5]	SOMALDO P, FERDIANSYAH F A, JATI G, 等. Developing Smart COVID-19 Social Distancing Surveillance Drone using YOLO Implemented in Robot Operating System simulation environment[C/OL]//2020 IEEE 8th R10 Humanitarian Technology Conference (R10-HTC). 2020: 1-6. DOI:10.1109/R10-HTC49770.2020.9357040.

[6]	钭锦周, 陈正东, 林玮, 等. 无人机海上风电机组叶片缺陷自适应无损检测[J]. 2025.

[7]	孙广宇. 基于电磁超声体波的铝板缺陷检测[D]. 哈尔滨工业大学, 2025.

[8]	涂君, 蔡卓越, 张旭, 等. 磁力可控电磁超声换能器设计及特性[J/OL]. 机械工程学报, 2021, 57(2): 46-52. DOI:10.3901/JME.2021.02.046.

[9]	CHEN W, LU C, LI X, 等. A novel laser-EMAT ultrasonic longitudinal wave resonance method for wall thickness measurement at high temperatures[J/OL]. Ultrasonics, 2024, 141: 107340. DOI:10.1016/j.ultras.2024.107340.

[10]	KUBRUSLY A C, KANG L, DIXON S. Measurement of Plate Thickness Based on the Optimal Unidirectional Generation of Ultrasonic Waves Using Dual Electromagnetic Acoustic Transducer[J/OL]. IEEE Transactions on Instrumentation and Measurement, 2023, 72: 1-9. DOI:10.1109/TIM.2023.3281566.

[11]	郭伟, 石文泽, 卢超, 等. 金属薄板厚度激光电磁超声谐振测量方法[J]. 航空学报, 2024.

[12]	苟浩瑞, 杜昱錡, 王思宇, 等. 一种磁场优化结构电磁超声传感器设计[J]. 2024.

[13]	PEI C, ZHAO S, XIAO P, 等. A modified meander-line-coil EMAT design for signal amplitude enhancement[J/OL]. Sensors and Actuators A: Physical, 2016, 247: 539-546. DOI:10.1016/j.sna.2016.07.006.

[14]	GONZÁLEZ-DESANTOS L M, MARTÍNEZ-SÁNCHEZ J, GONZÁLEZ-JORGE H, 等. Payload for Contact Inspection Tasks with UAV Systems[J/OL]. Sensors, 2019, 19(17): 3752. DOI:10.3390/s19173752.

[15]	KOCER B B, TJAHJOWIDODO T, PRATAMA M, 等. Inspection-while-flying: An autonomous contact-based nondestructive test using UAV-tools[J/OL]. Automation in Construction, 2019, 106: 102895. DOI:10.1016/j.autcon.2019.102895.

[16]	OLLERO A, HEREDIA G, FRANCHI A, 等. The AEROARMS Project: Aerial Robots with Advanced Manipulation Capabilities for Inspection and Maintenance[J/OL]. IEEE Robotics & Automation Magazine, 2018, 25(4): 12-23. DOI:10.1109/MRA.2018.2852789.

[17]	TRUJILLO M Á, MARTÍNEZ-DE DIOS J R, MARTÍN C, 等. Novel Aerial Manipulator for Accurate and Robust Industrial NDT Contact Inspection: A New Tool for the Oil and Gas Inspection Industry[J/OL]. Sensors, 2019, 19(6): 1305. DOI:10.3390/s19061305.

[18]	RASHAD R, BICEGO D, JIAO R, 等. Towards Vision-Based Impedance Control for the Contact Inspection of Unknown Generically-Shaped Surfaces with a Fully-Actuated UAV[C/OL]//2020 IEEE/RSJ International Conference on Intelligent Robots and Systems (IROS). Las Vegas, NV, USA: IEEE, 2020: 1605-1612. DOI:10.1109/IROS45743.2020.9341203.

[19]	WATSON R J, PIERCE S G, KAMEL M, 等. Deployment of Contact-Based Ultrasonic Thickness Measurements Using Over-Actuated UAVs[C/OL]//2020 European Workshop on Structural Health Monitoring (EWSHM). Springer, 2021. DOI:10.1007/978-3-030-64594-6_66.

[20]	MULLER A M, THURNHERR C, RÖSSLER L, 等. UACIS: Uncrewed Aerial Concrete Inspection System[J/OL]. e-Journal of Nondestructive Testing, 2022, 27(09). DOI:10.58286/27305.

[21]	ROBERT WATSON, KAMEL M, ZHANG D, 等. Dry Coupled Ultrasonic Non-Destructive Evaluation Using an Over-Actuated Unmanned Aerial Vehicle[J/OL]. IEEE Transactions on Automation Science and Engineering, 2022, 19(4): 2874-2889. DOI:10.1109/TASE.2021.3094966.

[22]	MARCELLINI S, D’ANGELO S, DE CRESCENZO A, 等. Development of a semi-autonomous framework for NDT inspection with a tilting aerial platform[C]//Experimental Robotics（ISER 2023）: 卷 30. Springer, 2024.

[23]	HE G. Enhancing the Physical Capability of Aerial Robots: From Inspection to Manipulation[D]. Carnegie Mellon University, 2025.

[24]	JAMOLLI K, KELLER D, SAMUEL P. Non-destructive testing (“ndt”) device particularly for unmanned aerial vehicle (“uav”), uav and kit for ndt comprising the device: WO2025186737A1[P]. 2025-09-12.

[25]	YOUNG K L, PHILLIPS C K, REYES M D, 等. Mission Results: Creating a 3D Map of a Very High Radiation Confined Space Using the LiDAR-Equipped Elios 3 Drone[J/OL]. Nuclear Science and Engineering, 2025, 199(8): 1351-1363. DOI:10.1080/00295639.2024.2364463.

[26]	VEENSTRA R, BAZZANA B, SMITS S, 等. Autonomous Aerial Non-Destructive Testing: Ultrasound Inspection with a Commercial Quadrotor in an Unstructured Environment[A/OL]. arXiv, 2026. DOI:10.48550/arXiv.2603.04642.

[27]	LIU Y, WANG S, SHI J, 等. Online Continual Physics-Informed Learning for Quadrotor State Estimation Under Wind-Induced Disturbances[J/OL]. Aerospace, 2025, 12(8): 704. DOI:10.3390/aerospace12080704.

[28]	FAKHRO K, PYRKIN A A. Learning Quadrotor Dynamics Using Feed-Forward Neural Networks[J/OL]. IFAC-PapersOnLine, 2025, 59(14): 179-183. DOI:10.1016/j.ifacol.2025.12.146.

[29]	SANYAL S, ROY K. RAMP-Net: A Robust Adaptive MPC for Quadrotors via Physics-informed Neural Network[C/OL]//2023 IEEE International Conference on Robotics and Automation (ICRA). 2023: 1019-1025. DOI:10.1109/ICRA48891.2023.10161410.

[30]	GU W, PRIMATESTA S, RIZZO A. Physics-informed Neural Network for Quadrotor Dynamical Modeling[J/OL]. Robotics and Autonomous Systems, 2024, 171: 104569. DOI:10.1016/j.robot.2023.104569.

[31]	ZHAO Z, DING X, PRAKASH B A. PINNsFormer: A Transformer-Based Framework For Physics-Informed Neural Networks[C]//The Twelfth International Conference on Learning Representations. 2023.

[32]	XIE Y, GONG Y, ZHAO Y, 等. MPFusionNet: Transformer-Based Multimodal Perception Fusion for Predictive Beamforming in Low-Altitude UAV Communication Networks[J/OL]. IEEE Internet of Things Journal, 2026, 13(4): 5517-5530. DOI:10.1109/JIOT.2025.3623880.